EP 100 WALL TIE MOUNT

Specification

Part 1 – General

  - Work Included

    - Supply of material, tools, and other equipment necessary to carry out the installation of the acoustic walls as shown on drawings.

  - System Description

- The acoustic walls shall be elastically suspended from the building structure, via acoustic wall mounts. The acoustic wall tie equipped with long ring-attached screw shall be threaded onto wooden or plastified walls of the building structure while the acoustic wall is fixed to a vertical metallic beam. The wall ties are conceived for withstanding horizontal load in (both senses) at a maximum load of 10Kg. Wall ties are not designed to withstand significant vertical loads. The weight of the drywall should be suspended completely on an elastic element below the wall.

  - Quality

    - The manufacturing of the wall tie mounts shall be performed in a way that it fulfills its function as well as to comply with all applicable regulations.

  - Submittals

    - Product CAD drawings shall be provided to the designer for approval.

    - Product performance data shall be submitted to the designer for review and shall include an Airborne Sound Insulation Report according to ISO 10140-2 standard. The tests shall be performed in an independent laboratory. Test reports shall document a minimum Sound Transmission Class (STC) for a resiliently suspended wall.

Part 2 – Products

2.01 Materials

- The acoustic wall tie mounts shall be designed and manufactured by AMC MECANOCAUCHO®.

Part 3 – Execution

3.01 Installation

- In places where rigid contact between the soundproofed wall and other non-elastically

suspended elements are present, elastic materials shall be used to avoid/eliminate the rigid connection that act as acoustic bridges.

- The installation of all sound isolation materials specified herein, shall be in accordance with procedures submitted by the isolation material manufacturer and approved by the architect.