EP 600 + SYLOMER® WALL TIE MOUNT

Specification

Part 1 – General

  - Work Included

    - Supply of material, tools, and other equipment necessary to carry out the installation of the acoustic walls as shown on drawings.

  - System Description

- The acoustic walls shall be elastically suspended from the building structure, via acoustic wall mounts. The metal bracket of each wall tie mount shall be fixed to the building structure while the acoustic wall is fixed to a vertical metallic beam. The wall ties are conceived for withstanding horizontal load in (both senses) at a maximum load of 30Kg. Wall ties are not designed to withstand significant vertical loads. The weight of the drywall should be suspended completely on an elastic element below the wall.

  - Quality

    - The manufacturing of the wall tie mounts shall be performed in a way that it fulfills its function as well as to comply with all applicable regulations.

  - Submittals

    - Product CAD drawings shall be provided to the designer for approval.

    - Product performance data shall be submitted to the designer for review and shall include an Airborne Sound Insulation Report according to ISO 10140-2 standard. The tests shall be performed in an independent laboratory. Test reports shall document a minimum Sound Transmission Class (STC) for a resiliently suspended wall.

Part 2 – Products

2.01 Materials

- The acoustic wall tie mounts shall be designed and manufactured by AMC MECANOCAUCHO®.

- The acoustic wall ties shall be manufactured following the fail-safe concept, even if the elastic element disappears completely, the acoustic wall ties will keep its structural integrity.

- The bracket of the acoustic wall tie shall be designed to allow for 10 degrees of horizontal misalignment without making metal-to-metal contact between the stud bolt and the bracket.

Part 3 – Execution

3.01 Installation

- In places where rigid contact between the soundproofed wall and other non-elastically

suspended elements are present, elastic materials shall be used to avoid/eliminate the rigid connection that act as acoustic bridges.

- The installation of all sound isolation materials specified herein, shall be in accordance with procedures submitted by the isolation material manufacturer and approved by the architect.